ERROR ANALYSIS TASK

34. A rational sampling rate conversion algorithm applies interpolation after decimation. Using an input sampled at 24 kHz with L = 2 and M = 3, analyze why aliasing occurs and propose the correct processing order.

Rational Sampling Rate Conversion:

Error Analysis: Wrong Processing Order (Interpolation after Decimation):

Given

- Input Sampling Frequency = 24 kHz

- Interpolation Factor (L) = 2

- Decimation Factor (M) = 3

Sampling:

- Sampling converts an analog signal into a digital signal.

- Sampling frequency (Fs) = Number of samples taken per second.

Example:

- Fs = 24 kHz

- Means 24,000 samples are taken every second.

## Nyquist Frequency:

- Imagine a car moving at 100 km/h.

- If you take one photo every 10 seconds, you miss most of the motion.

- Similarly, if a signal changes very quickly but we take too few samples, we lose information.

- So, we need a minimum sampling rate.

- This is where the Nyquist Frequency comes in.

### Interpretation:

Interpolation is the process of increasing the sampling rate by adding new samples between the existing samples.

It is also called Upsampling.

- Then a low-pass interpolation filter calculates suitable values for those new positions.

- Original samples remain.

- New samples are added between them.

Decimation:

Decimation is the process of decreasing the sampling rate by removing some samples.

It is also called Downsampling.

- Aliasing

- Suppose the input contains a 6 kHz signal.

- Before Decimation

- Nyquist = 12 kHz

- 6 < 12

- Safe

- After Decimation

- Sampling = 8 kHz

- Nyquist = 4 kHz

- 6 > 4

- Aliasing Occurs

Why Interpolation Cannot Fix Aliasing:

Wrong order:

24 kHz   > Decimation >8 kHz >Aliasing >Interpolation >16 kHz

Interpolation cannot recover the original signal.

Correct order:

Input (24 kHz) >Interpolation (×2) >48 kHz >Low Pass Filter >Decimation (÷3) >16 kHz

24×2/3=16kHz

Conclusion:
Decimation to Interpolation:

- Sampling rate decreases first.

- Nyquist frequency becomes smaller.

- Aliasing occurs.

- Interpolation cannot remove aliasing.

Interpolation to Low Pass Filter to Decimation:

- Input Sampling Rate = 24 kHz

- Output Sampling Rate = 16 kHz

- Aliasing is avoided.